Specification_12V System

| MODEL | AC 0212A | AC 0512A | AC 061 2A | AC 0712A | AC 1212A |
| --- | --- | --- | --- | --- | --- |
| Input Voltage (Vac) | 100-240 |  | 100/220 | ± 10% Auto Switch |  |
| Input Frequency | 50 / 60 Hz |  |  |  |  |
| Output Current | 2A | 3A 4 | A 5A | 6A 7A 8 | A 10A 12A |
| Output Equalizer Voltage | 14.6 Vdc ± 0.2 V ( Standard Setting ) |  |  |  |  |
| Output Float Voltage | 13.5 Vdc ± 0.2 V ( standard Setting ) |  |  |  |  |
| Battery Application | Lead Acid Battery |  |  |  |  |
| Dimensions (L x W x H) mm | 120x61.6x37.4 | 186 x 88 x 47.5 |  |  |  |
| Weight | 0.5 kgs | 0.8 Kgs |  |  |  |

Specification_24V System

| MODEL | AC 0224A | AC 0324A | AC 0424A AC 0524A | AC 0624A AC 0724A |
| --- | --- | --- | --- | --- |
| Input Voltage | 100-240Vac | 100V/ 220Vac ± 10% Auto Switch |  |  |
| Input Frequency | 50 / 60Hz |  |  |  |
| Output Current | 2A | 3A | 4A 5A ; | <• 6A 7 A |
| Output Equalizer Voltage | 29.2 Vdc ± 0.2 V ( Standard Setting ) |  |  |  |
| Output Float Voltage . | 27.0 Vdc ± 0.2 V ( Standard Setting ) |  |  |  |
| Battery Application | Lead Acid Battery |  |  |  |
| Dimensions (L x W x H) mm | 120* 61.6x37.4 | 186 x 88 x 47.5 |  |  |
| Weight | 0.5 kgs | 0.8 Kgs |  |  |

| Problem and Symptoms | Possible Reason | Solution |
| --- | --- | --- |
| If power LED indicator is off and no output voltage, when charger plug in socket. | 1. No AC input 2. Charger malfunction | 1. Check AC power source. 2. Notify service center |
| If commercial power is normal. But charging indicate LED (Equalizer) is off when charging. • | 1 . The polarity connection maybe reversed. 2. The output connection maybe short or open. 3. Output DC fuse loose (for 3A-12A) 4. Charger malfunction | 1 . Check output cable 2. Check output cable 3. Check output DC fuse (for 3A-12A) 4. Notify service center |
| If commercial power is normal. But charging indicate LED (Equalizer) is light for a long time. | 1. Battery malfunction 2. Charger malfunction | 1. The battery can not charge normally, please stop charging and have battery replaced. 2. Notify service center |
| If the charging Indicator from off turns to on (fully charged) immediately. | 1 .Battery fully charged. 2. Battery malfunction. | It means the battery is not fully charged and it may be defective. Please have battery replaced. |